# SOA CAN Module Architecture and Design

## Version History

| Number | Date | Author | Approver | Revision History | Status |
| --- | --- | --- | --- | --- | --- |
| V0.1 | 2026.4.27 | weizhichun |  | create initial version | draft |
| V0.1 | 2026.4.28 | weizhichun | Li Yijie | Reviewd by lisong and Liyijie | Approved |

This document describes the current SOA CAN module architecture in the M-Core project. The focus is the active software path in the current code, not a future target architecture.

## 1. System Overview

The SOA CAN module is responsible for cyclic transmission and reception on FlexCAN0. In the current project, the active path is:

- PreOS initialization in main() completes FlexCAN controller startup and RX MB configuration.

- Runtime processing is driven by the unified 10 ms FreeRTOS task.

- DBC conversion is handled by the generated SOA_CANdbc_Generated layer.

- The current active cyclic function is AINFC_Can_Cyclic_10ms().

SoaAdapter_Init() is already called during initialization, but SoaAdapter_Main() is not scheduled in the current runtime path.

## 2. Module Architecture

The current CAN module can be understood as three layers:

- Application Data Layer

  - Uses the generated global structures g_tx_Standard_100_Tx and g_rx_Standard_200_Rx.

- CAN Service Layer

  - AINFC_Can_Cyclic_10ms() drives cyclic RX/TX.

  - FlexCAN_Message_Tx_pack() and FlexCAN_Message_Rx_unpack() connect DBC conversion with CAN send/receive.

  - AINFC_Can_TxMsg() and AINFC_Can_RxMsgL() provide the direct FlexCAN wrapper interface.

- Hardware Layer

  - NXP RTD FlexCAN_Ip APIs access FlexCAN0 message buffers.

## 3. Runtime Flow

### 3.1 Initialization Flow

The CAN-related startup sequence in main() is:

- FlexCAN_Ip_Init()

- FlexCAN_Ip_SetStartMode()

- FlexCAN_Process_Init()

- PICC_PreOS_Init()

- SoaAdapter_Init()

- OsTask_Creation_All()

This means FlexCAN0 is fully initialized before the scheduler starts.

### 3.2 10ms Cyclic Flow

At runtime, the CAN module is executed from TASK_M0_10MS() inside the unified OS_10ms thread.

The current code executes this path every 10 ms.

## 4. Message and MB Mapping

| Direction | MB | CAN ID | Usage |
| --- | --- | --- | --- |
| TX | MB0 | 0x100 | DBC TX message, source is g_tx_Standard_100_Tx |
| TX | MB1 | 0x101 | Raw debug frame |
| RX | MB2 | 0x200 | DBC RX message, target is g_rx_Standard_200_Rx |
| RX | MB3 | 0x201 | Raw RX frame |

The current generated DBC layer only covers:

- 0x100 TX

- 0x200 RX

0x101 and 0x201 are not DBC messages in the current implementation.

## 5. Key Components and Files

- src/Ecum_Init_Main/EcuM_main_init.c

  - CAN startup entry.

  - Calls FlexCAN initialization and FlexCAN_Process_Init().

- src/Task/Ostask_main.c

  - Runtime scheduling entry.

  - Calls AINFC_Can_Cyclic_10ms() from TASK_M0_10MS().

- COM/FlexCAN_Ip/FlexCAN_Ip_main.c / .h

  - Main CAN business logic.

  - Provides MB mapping, RX/TX wrapper functions and cyclic processing.

- SWC/Soa_Adapter/SOA_CANdbc_Generated.c / .h

  - Generated DBC pack/unpack layer.

  - Provides the global TX/RX message structures.

- SWC/Soa_Adapter/SOA_Adapter.c / .h

  - Current SOA registration layer.

  - SoaAdapter_Init() is active, but SoaAdapter_Main() is not part of the current runtime flow.

## 6. Current Implementation Notes

- The active CAN runtime path is TASK_M0_10MS() -> AINFC_Can_Cyclic_10ms().

- SoaAdapter_Main() is not scheduled in the current code.

- The generated DBC metadata defines 0x100 and 0x200 as 100 ms messages, but the current runtime path executes every 10 ms.

- 0x100 is generated as a 2-byte DBC message, but the current FlexCAN TX data info is configured with DLC 8.

- FlexCAN_GetBusDiagStatus() exists as a diagnostic interface, but it is not yet integrated into the periodic runtime path.