# SOA Adapter Module Design Document

| Number | Date | Author |  | Approver | Revision History | Status |
| --- | --- | --- | --- | --- | --- | --- |
| V0.1 | 2026.5.18 |  | weizhichun |  | create initial version | draft |
| V0.1 | 2026.5.21 |  | weizhichun | Li Yijie | Reviewed by lisong and Liyijie | Approved |

| Item | Description |
| --- | --- |
| Module Name | SOA Adapter |
| Target Platform | S32G399A M7 Core (FreeRTOS) |

## 1. Overview

SOA Adapter is the SOA (Service-Oriented Architecture) adaptation layer module on the M-Core, responsible for bridging CAN signal data from within the M-Core to the SOA service framework on the A-Core. This module communicates with the A-Core through the PICC middleware via the IPCF shared memory channel.

Important Note : soa_adapter_cnf.c  and soa_adapter_cnf.h is generated by soa_adapter_generator.py ,the input file is CANdbc_file.dbc  and S2S_HPC_A_SAG.xlsx,so  you should check CANdbc_file.dbc  and S2S_HPC_A_SAG.xlsx carefully ;

Data Flow:

CAN Bus <-> DBC Structs <-> SOA Adapter <-> PICC API <-> IPCF <-> A-Core

## 2. Architecture Design

### 2.1 Module Composition

| File | Responsibility |
| --- | --- |
| soa_adapter_main.h | Public interface declarations |
| soa_adapter_main.c | Core logic implementation (serialization, Notifier transmission, Method processing, main loop) |
| soa_adapter_cnf.h | Configuration definitions (SOA Header structure, service matrix, ID macros) |
| soa_adapter_cnf.c | Configuration instantiation (signal read/write functions, service configuration table) |

### 2.2 External Dependencies

| Dependency Module | Purpose |
| --- | --- |
| picc_api.h | PICC driver public API (Init, SendEvent, MethodResponse, GetMethodData, etc.) |
| FlexCAN_Ip_main.h | CAN transmit/receive interface |
| SOA_CANdbc_Generated.h | DBC-generated CAN signal global structs |
| Platform.h | Platform base type definitions (uint8, uint16, sint8, boolean, etc.) |

### 2.3 PICC Registration Configuration

localId    = 71 (0x47)    // M-Core SOA Provider ID (ID range 71-80)
remoteId   = 76 (0x4C)    // A-Core SOA Consumer ID
role       = PICC_ROLE_SERVER
channelId  = 2            // IPCF Channel 2
appIndex   = PICC_APP_SOA (7)
mode       = Polling      // methodHandler = NULL, eventHandler = NULL

## 3. Protocol Format

### 3.1 Two-Layer Protocol Structure

┌──────────────────┬──────────────────────────────────────┐
│  IPC 8B Header   │            IPC Payload               │
│ (PICC layer)     │         (SOA Adapter layer)          │
│                  ├──────────────────┬───────────────────┤
│                  │  SOA 12B Header  │  SOA Actual Data  │
└──────────────────┴──────────────────┴───────────────────┘

### 3.2 SOA Header (12 Bytes, Big-Endian)

| Byte Offset | Field | Length | Description |
| --- | --- | --- | --- |
| 0-1 | SOA_ServiceID | 2B | AP Service ID |
| 2-3 | SOA_MethodID | 2B | AP Method/Event ID |
| 4-5 | SOA_InstanceID | 2B | AP Instance ID |
| 6-7 | SOA_SessionID | 2B | Session ID (fixed at 0 for Notifier; echoed from request for Getter/Setter) |
| 8-9 | SOA_ReturnCode | 2B | Return code (0 = success, non-zero = failure) |
| 10-11 | SOA_Length | 2B | Length of the subsequent actual parameter data |

Corresponding Code Struct:

typedef struct {
    uint16 SOA_ServiceID;
    uint16 SOA_MethodID;
    uint16 SOA_InstanceID;
    uint16 SOA_SessionID;
    uint16 SOA_ReturnCode;
    uint16 SOA_Length;
} SOA_Header_t;

### 3.3 IPC Layer Fixed ID Mapping

| SOA Business Type | IPC Layer ID | IPC MessageType |
| --- | --- | --- |
| All Notifiers/Events (M→A) | EventID = 3 | 0x09 (NOTIFICATION_WITHOUT_ACK) |
| All Getters/Setters/Methods (A→M→A) | MethodID = 1 | 0x05 (REQUEST) → 0x80 (RESPONSE) |

## 4. Service Matrix

### 4.1 Service Configuration Table

| Index | Service Interface Name | ServiceID | MethodID | InstanceID | Type | Data Source | Size |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Atom_VCU_DriSpeedSt | 0x0001 | 0x8001 | 0x0001 | Notifier | g_rx_Standard_200_Rx.VehicleSpeed | 2B |
| 1 | Atom_VCU_ParkingSt | 0x0002 | 0x5001 | 0x0001 | Getter | g_rx_Standard_200_Rx.ParkingSts | 1B |
| 2 | Atom_VCU_HighVoltageBatterySt | 0x0003 | 0x5001 | 0x0001 | Getter | g_rx_Standard_200_Rx.HighVoltageBatterySts | 2B |
| 3 | Atom_VCU_IgnitionSt | 0x0004 | 0x5001 | 0x0001 | Getter | g_tx_Standard_100_Tx.IgnitionSts | 1B |
| 4 | Atom_BCM_VehicleMode | 0x0005 | 0x5001 | 0x0001 | Setter | g_tx_Standard_100_Tx.VehicleMode | 1B |
| 5 | Atom_BCM_VehicleModeSt | 0x0005 | 0x8001 | 0x0001 | Notifier | g_tx_Standard_100_Tx.VehicleMode | 1B |

### 4.2 Setter-Notifier Linkage

Entry[4] (Setter) and Entry[5] (Notifier) share the same ServiceID=0x0005: - hasLinkedNotifier = TRUE - linkedNotifierIdx = 5 - After a successful Setter write, the linked Notifier's current value is read via Entry[5]'s readFunc and placed into the RESPONSE

### 4.3 Notifier Index Table

g_soaNotifierIndices[SOA_NOTIFIER_COUNT=2] = { 0, 5 };
// [0] → DriSpeedSt Notifier (ServiceTable[0])
// [1] → VehicleModeSt Notifier (ServiceTable[5])

## 5. Core Data Structures

### 5.1 Service Configuration Entry

typedef struct {
    uint16              SOA_ServiceID;
    uint16              SOA_MethodID;
    uint16              SOA_InstanceID;
    SOA_ServiceType_e   serviceType;       // NOTIFIER / GETTER / SETTER
    SOA_SignalReadFunc_t  readFunc;         // Signal value read function pointer
    SOA_SignalWriteFunc_t writeFunc;        // Signal value write function pointer
    uint16              SOA_EventGroupID;
    uint8               dataSize;
    boolean             hasLinkedNotifier;
    uint8               linkedNotifierIdx;
} SOA_ServiceConfig_t;

### 5.2 Notifier Change Detection Cache

typedef struct {
    uint8   prevData[SOA_MAX_DATA_SIZE];  // Previous signal value
    uint16  prevLen;                       // Previous data length
    boolean isValid;                       // TRUE after first read
} SOA_NotifierCache_t;

### 5.3 Module State

typedef struct {
    boolean             isInitialized;
    PICC_LinkState_e    prevLinkState;                      // Link state edge detection
    SOA_NotifierCache_t notifCache[SOA_NOTIFIER_COUNT];     // Change detection cache
} SOA_AdapterState_t;

### 5.4 Key Constants

| Macro | Value | Description |
| --- | --- | --- |
| SOA_HEADER_SIZE | 12 | SOA protocol header size (bytes) |
| SOA_MAX_DATA_SIZE | 256 | Maximum payload per single message |
| SOA_MAX_MSG_SIZE | 268 | Header + Data |
| SOA_SERVICE_TABLE_COUNT | 6 | Total number of services |
| SOA_NOTIFIER_COUNT | 2 | Number of Notifier services |

## 6. Functional Flow

### 6.1 Initialization Flow (SoaAdapter_Init)

flowchart TD
    A[SoaAdapter_Init] --> B[memset zero s_soaState]
    B --> C[prevLinkState = DISCONNECTED]
    C --> D[PICC_Init: Register PICC_APP_SOA]
    D --> E[isInitialized = TRUE]

Integration Point: Called within App_Init_All() in EcuM_main_init.c.

### 6.2 10 ms Main Loop (SoaAdapter_Main)

flowchart TD
    A["SoaAdapter_Main (10ms)"] --> B{isInitialized?}
    B -- No --> Z[Return]
    B -- Yes --> C[Get App Link State]
    C --> D{DISCONNECTED→CONNECTED?}
    D -- Yes --> E[CAN RX Refresh]
    E --> F[Send All Notifier Initial Values]
    D -- No --> G{Link Connected?}
    G -- No --> H[Save prevLinkState]
    H --> I{DISCONNECTED?}
    I -- Yes --> J[Invalidate Notifier Cache]
    I -- No --> Z
    J --> Z
    G -- Yes --> K[Save prevLinkState]
    K --> L[CAN RX Processing]
    L --> M[Detect Notifier Changes and Send]
    M --> N[Poll Method Requests]
    N --> O[CAN TX Processing]
    O --> Z

### 6.3 Notifier Transmission Flow

#### 6.3.1 Initial Value Synchronization (SOA_SendAllNotifierInitValues) — Batched Transmission

- Trigger Condition: Link transitions from DISCONNECTED to CONNECTED

- Behavior: Iterate over g_soaNotifierIndices, concatenate each Notifier's SOA message (12B Header + Data) into s_soaBatchBuf, then send all initial values with a single PICC_SendEvent() call

- Batch Format: [SOA_Header(12B) + Data_1] [SOA_Header(12B) + Data_2] ... [SOA_Header(12B) + Data_N]

- Simultaneously: Initialize the change detection cache (prevData, prevLen, isValid=TRUE)

- Efficiency: Reduces N PICC_SendEvent calls to 1, lowering IPC call overhead

#### 6.3.2 Change Detection Transmission (SOA_CheckAndSendNotifiers)

- Period: 10 ms

- Behavior: Read the current value of each Notifier, compare against cache (memcmp); if changed, send and update cache

#### 6.3.3 Single Notifier Construction (SOA_SendNotifier)

- Read signal value via readFunc into s_soaTxBuf[12..]

- Build SOA Header (SessionID=0, ReturnCode=0)

- Serialize Header into s_soaTxBuf[0..11]

- Call PICC_SendEvent(PICC_APP_SOA, EventID=3, buf, len, WITHOUT_ACK)

#### 6.3.4 SOA-Level Batching

Per the SOA protocol specification: Multiple Event/Notifier 14-byte data bodies can be concatenated into a single long Payload within the same underlying message for increased transmission efficiency.

Current Implementation Scope: SOA-level batching is used only during the initial value synchronization phase (SOA_SendAllNotifierInitValues). Periodic change-detection transmission (SOA_CheckAndSendNotifiers) still sends one message at a time.

Batch Data Format:

┌────────────────────────────────┬────────────────────────────────┬─────┐
│ SOA Msg 1 (12B Hdr + Data)    │ SOA Msg 2 (12B Hdr + Data)    │ ... │
└────────────────────────────────┴────────────────────────────────┴─────┘
                    ↓ Treat entire block as a single IPC Payload ↓
┌──────────────────┬──────────────────────────────────────────────┐
│  IPC 8B Header   │         Batched SOA Payload                 │
└──────────────────┴──────────────────────────────────────────────┘

Example (current 2-Notifier initial value sync):

SOA Msg 1: VehicleSpeed (14B = 12B header + 2B data)
  00 01  80 01  00 01  00 00  00 00  00 02  XX XX

SOA Msg 2: VehicleModeSt (13B = 12B header + 1B data)
  00 05  80 01  00 01  00 00  00 00  00 01  XX

→ Concatenated PICC_SendEvent payload length = 14 + 13 = 27 bytes

Key Constraints: - Total batch size must not exceed SOA_MAX_MSG_SIZE (268B) - A-Core must unpack message by message according to SOA_Header.SOA_Length during parsing - This batching is only concatenation of SOA-layer data; the underlying IPCF stacking (CRC enable bit + Counter + CRC16) is handled independently by the PICC Stack layer

### 6.4 Method Request Processing Flow

#### 6.4.1 Polling (SOA_PollMethodRequests)

- Call PICC_GetMethodData(PICC_APP_SOA, MethodID=1, ...) to check for pending requests

- Upon receiving data, call SOA_HandleMethodRequest()

#### 6.4.2 Request Handling (SOA_HandleMethodRequest)

flowchart TD
    A[Receive Method Request] --> B[Deserialize SOA Header]
    B --> C[Look Up Service Table]
    C --> D{Found?}
    D -- No --> E[RESPONSE: ReturnCode=1]
    D -- Yes --> F{serviceType?}
    F -- GETTER --> G[readFunc Read Current Value]
    G --> H{Read Successful?}
    H -- Yes --> I[Fill RESPONSE Data]
    H -- No --> J[ReturnCode=1]
    F -- SETTER --> K[writeFunc Write Value]
    K --> L{Write Successful?}
    L -- No --> M[ReturnCode=1]
    L -- Yes --> N{hasLinkedNotifier?}
    N -- Yes --> O[Read Linked Notifier Current Value into RESPONSE]
    N -- No --> P[RESPONSE Length=0]
    F -- NOTIFIER --> Q[ReturnCode=1, Should Not Reach This Branch]
    I --> R[PICC_MethodResponse]
    J --> R
    M --> R
    O --> R
    P --> R
    Q --> R

Key Rules: - RESPONSE must echo the SOA_SessionID from the request - Setter+Notifier linkage: after a successful write, read the linked Notifier's current value as the RESPONSE data - Setter without linkage: return an empty RESPONSE with Length=0

## 7. Link State Management

### 7.1 State Machine

DISCONNECTED ──(A-Core Server replies with connection confirmation)──> CONNECTED
CONNECTED ──(A-Core sends disconnect notification / heartbeat timeout)──> DISCONNECTED

### 7.2 SOA Adapter Link-Aware Behavior

| State Transition | SOA Adapter Behavior |
| --- | --- |
| DISCONNECTED → CONNECTED | Refresh CAN RX → Send all Notifier initial values → Initialize change detection cache |
| CONNECTED (steady state) | Normal Notifier change detection + Method polling |
| → DISCONNECTED | Stop business transmission, invalidate all Notifier caches (isValid=FALSE) |
| Re-CONNECTED | Send all Notifier initial values again (guarantees A-Core state synchronization) |

### 7.3 M-Core Constraints

- Before link establishment: Prohibit sending any SOA business data

- On disconnection: Immediately stop SOA business message transmission

- Only send WITHOUT_ACK Events (M-Core real-time constraint)

## 8. Signal Read/Write Functions

### 8.1 Read Functions (Notifier + Getter)

| Function | Signal Source | Serialization | Return Length |
| --- | --- | --- | --- |
| SOA_ReadVehicleSpeed | g_rx_Standard_200_Rx.VehicleSpeed | uint16 big-endian | 2B |
| SOA_ReadWorkVehicleMode | g_tx_Standard_100_Tx.VehicleMode | uint8 | 1B |
| SOA_ReadParkingSts | g_rx_Standard_200_Rx.ParkingSts | uint8 | 1B |
| SOA_ReadHighVoltageBatterySts | g_rx_Standard_200_Rx.HighVoltageBatterySts | uint16 big-endian | 2B |
| SOA_ReadIgnitionSts | g_tx_Standard_100_Tx.IgnitionSts | uint8 | 1B |

### 8.2 Write Functions (Setter)

| Function | Signal Target | Input | Return |
| --- | --- | --- | --- |
| SOA_WriteVehicleMode | g_tx_Standard_100_Tx.VehicleMode | 1B uint8 | 0=success, 1=failure |

### 8.3 Function Signatures

typedef uint16 (*SOA_SignalReadFunc_t)(uint8 *outBuf, uint16 maxLen);
typedef uint8  (*SOA_SignalWriteFunc_t)(const uint8 *inBuf, uint16 len);

## 9. Static Memory Allocation

| Variable | Type | Size | Purpose |
| --- | --- | --- | --- |
| s_soaState | SOA_AdapterState_t | ~518B | Module state + Notifier cache |
| s_soaTxBuf | uint8[] | 268B | Buffer for building individual Notifier transmissions |
| s_soaBatchBuf | uint8[] | 268B | Batch buffer for initial value synchronization |
| s_methodRxBuf | uint8[] | 268B | Method request receive buffer |
| s_methodRspBuf | uint8[] | 268B | Method response build buffer |

Total static RAM usage approximately 1.6 KB

## 10. CAN Signal Mapping

### 10.1 CAN RX (Receive)

| CAN ID | DBC Struct | Signals Used by SOA |
| --- | --- | --- |
| 0x200 | g_rx_Standard_200_Rx | VehicleSpeed, ParkingSts, HighVoltageBatterySts |

### 10.2 CAN TX (Transmit)

| CAN ID | DBC Struct | Signals Used by SOA |
| --- | --- | --- |
| 0x100 | g_tx_Standard_100_Tx | VehicleMode, IgnitionSts |

## 11. Message Examples

### 11.1 Notifier: VehicleSpeed = 120 km/h (0x0078)

SOA Payload (12+2=14 bytes):

00 01  80 01  00 01  00 00  00 00  00 02  00 78
│ServiceID│MethodID│InstanceID│SessionID│ReturnCode│ Length │ Data │

IPC Layer Call:

PICC_SendEvent(PICC_APP_SOA, 3, payload, 14, PICC_EVENT_WITHOUT_ACK);

### 11.2 Getter: A-Core Requests ParkingSts

A→M Request SOA Payload:

00 02  50 01  00 01  00 03  00 00  00 00
│ServiceID│MethodID│InstanceID│SessionID│ReturnCode│Length=0│

M→A Response SOA Payload (ParkingSts=2):

00 02  50 01  00 01  00 03  00 00  00 01  02
│ServiceID│MethodID│InstanceID│SessionID│ReturnCode│Length=1│Data│

### 11.3 Setter: A-Core Sets VehicleMode=1 (with Linked Notifier)

A→M Request:

00 05  50 01  00 01  00 05  00 00  00 01  01

M→A Response (reads linked Notifier current value=1):

00 05  50 01  00 01  00 05  00 00  00 01  01

## 12. Integration Notes

### 12.1 Initialization Sequence

1. PICC_PreOS_Init()          // IPCF driver + PICC infrastructure
2. FlexCAN_Process_Init()     // CAN driver initialization
3. SoaAdapter_Init()          // SOA Adapter registration (in App_Init_All())
4. vTaskStartScheduler()      // Start RTOS

### 12.2 Periodic Task

TASK_M0_10MS() {
    ...
    SoaAdapter_Main();    // Called every 10 ms
    ...
}

## 13. Extension Guide

### 13.1 Steps for Adding a New Signal

- soa_adapter_cnf.h: Add ServiceID/MethodID macros and index macros; update SOA_SERVICE_TABLE_COUNT

- soa_adapter_cnf.c: Implement Read/Write functions; add entry to g_soaServiceTable

- If it is a Notifier: update SOA_NOTIFIER_COUNT and g_soaNotifierIndices

- If it is a Setter+Notifier linkage: set hasLinkedNotifier=TRUE and linkedNotifierIdx

### 13.2 Design Constraints

- M-Core only sends WITHOUT_ACK Events

- M-Core does not support synchronous waiting; all operations are asynchronous/polling

- Prohibit sending business data before link establishment

- All multi-byte fields use big-endian serialization

- Initial value synchronization uses SOA-level batching (multiple Notifiers concatenated into a single IPC Payload); periodic change detection sends one message at a time